HT_MK1 Harness Tester

Flashing Procedure

| Document ID: | HT_MK1_FP |
| --- | --- |
| Document Version: | 1.0 |
| Date: | 27 August 2026 |
| System: | Control Card (STM32G474RET6) |
| Prepared by: | Azista BST Aerospace Private Limited |
| Prepared for: | Infinity Integration Pvt. Ltd. |
| Classification: | Confidential |

# Revision History

| Version | Date | Description | Author |
| --- | --- | --- | --- |
| 1.0 | 27 August 2026 | Initial release | [Author] |

# Table of Contents

Revision History	2

Table of Contents	2

1. Purpose and Scope	3

1.1 Purpose	3

1.2 Scope	3

1.3 Applies To	3

2. Prerequisites	3

2.1 Hardware	3

2.2 Software	3

3. Safety Precautions	3

4. Procedure — Building the Firmware	4

5. Procedure — Programming the Device	5

6. Verification	6

7. Troubleshooting	6

8. Sign-off	7

# 1. Purpose and Scope

## 1.1 Purpose

This procedure describes how to build and program (flash) the HT_MK1 firmware onto the Control Card's microcontroller.

## 1.2 Scope

Covers building the firmware in STM32CubeIDE and programming it through a debug probe connected to the Control Card. Does not cover firmware development, GUI operation, or functional testing — see the HT_MK1 Code Walkthrough, User Manual, and Functional Test Procedure for those.

## 1.3 Applies To

Control Card assemblies fitted with an STM32G474RET6 microcontroller.

# 2. Prerequisites

## 2.1 Hardware

| Item | Notes |
| --- | --- |
| Control Card assembly | STM32G474RET6 fitted, with the JTAG/SWD connector (J6) and the console Micro-USB-B connector accessible. |
| Debug probe: ST-Link/V2 | Standalone ST-Link/V2 dongle. Its 20-way ARM JTAG ribbon cable connects directly to J6 — no adapter needed. |
| USB cable: USB-A to Mini-USB-B | Connects the ST-Link/V2 (Mini-USB-B socket on the probe) to the PC (USB-A port). Used only for programming. |
| USB cable: USB-A to Micro-USB-B | Connects the Control Card's console connector directly to the PC (USB-A port). Used for the serial link — verification (Section 6) and normal GUI operation. |
| Programming PC | Runs the software listed in Section 2.2. |

NOTE:  The ST-Link/V2 and its Mini-USB-B cable are used for programming only (Section 5). The Micro-USB-B console connector is separate, external to the ST-Link/V2, and carries only the firmware-GUI serial link (Section 6) — it plays no part in flashing the device.

## 2.2 Software

STM32CubeIDE, version 1.19.0 or later — includes the bundled arm-none-eabi-gcc toolchain and ST-Link driver support.

The HT_MK1 firmware project source.

(Optional, for verification) A serial terminal application.

# 3. Safety Precautions

CAUTION:  Power down the harness tester fully before connecting or disconnecting the debug probe. Never program the Control Card while an HV Card is energized or a test is running.

Use standard ESD precautions (anti-static wrist strap, grounded mat) when handling the Control Card.

Jumpers JP1 and JP2, next to connector J6, must be in their as-designed position before connecting the probe. If unsure, check their position against the Control Card schematic before proceeding.

# 4. Procedure — Building the Firmware

1.	Open STM32CubeIDE and import the HT_MK1 project (File > Open Projects from File System, then select the project's root folder).

Figure 1 — The HT_MK1 project loaded in STM32CubeIDE's Project Explorer.

2.	Select the Debug build configuration — the only configuration this project currently provides.

3.	Build the project (Project > Build Project, or the hammer icon). A clean build compiles every source file with no errors.

Figure 2 — A successful build: 0 errors, 0 warnings.

Alternative — command line, from the project's Debug/ folder:

make all -j8

Requires the STM32CubeIDE bundled toolchain to be on PATH — see Section 7 if make cannot find arm-none-eabi-gcc.

# 5. Procedure — Programming the Device

1.	Connect the ST-Link/V2's 20-way ribbon cable to connector J6 on the Control Card.

2.	Connect the ST-Link/V2 to the PC using the USB-A to Mini-USB-B cable.

3.	Power on the Control Card.

4.	In STM32CubeIDE, start a debug session (Run > Debug, or the Debug icon). STM32CubeIDE programs the device automatically as part of starting the session.

Alternative: to flash a pre-built image without opening the full project, use STM32CubeProgrammer with the .elf file produced by the build (Debug/HT_MK1.elf) and the connected debug probe.

5.	Once programming completes, the microcontroller resets and starts running the new firmware automatically.

# 6. Verification

This step confirms the new firmware is running, using the console connector — a separate Micro-USB-B port on the Control Card, unrelated to the ST-Link/V2 and J6 used for flashing in Section 5. This port carries the same serial link the GUI uses during normal operation.

1.	Connect a USB-A to Micro-USB-B cable from the PC to the Control Card's console connector.

2.	On the PC, identify the new COM port and open a serial terminal on it at 115200 baud, 8 data bits, no parity, 1 stop bit (115200 8N1).

3.	Power-cycle or reset the Control Card.

4.	Confirm the console prints a boot line similar to:

#[boot] HT_MK1 console up @115200

5.	Send the command shown below and confirm the reply is received:

>PING
<PONG

NOTE:  A PONG reply confirms the new firmware is running and the serial link works. It does not confirm the Matrix Card or HV Cards are functioning — run the Functional Test Procedure for that.

COORDINATION POINT:  The console connector's UART must match the UART the firmware actually initializes for the console. Confirm this before relying on the check above — see the HT_MK1 Code Walkthrough, Section 3, for the firmware's current console UART assignment.

# 7. Troubleshooting

| Symptom | Likely Cause | What to Do |
| --- | --- | --- |
| "arm-none-eabi-gcc: command not found" (command-line build) | The bundled toolchain is not on PATH. | Add STM32CubeIDE's bundled toolchain folder to PATH, or build from within the IDE instead. |
| Linker error referencing a path that does not exist on this PC | The build folder's linker-script rule was generated on a different machine. | Clean and rebuild the project from within STM32CubeIDE so it regenerates the build folder locally. |
| Debug probe not detected | Cable, connector, or driver issue. | Confirm the ribbon cable is fully seated on J6, the probe's USB cable is connected, and its driver is installed. Confirm JP1/JP2 positions (Section 3). |
| No boot line, or no reply to PING | Wrong COM port, wrong baud rate, or programming did not complete. | Confirm the terminal is set to 115200 8N1 on the correct COM port. Re-run Section 5 and check STM32CubeIDE's console for errors. |

# 8. Sign-off

| Field | Value |
| --- | --- |
| Firmware version programmed |  |
| Programmed by |  |
| Date |  |
| Verified by |  |