HT_MK1 Harness Tester

Code Walkthrough

Document ID: HT_MK1_SDD

Revision: 1.0

Date: 26-Aug-2026

Infinity Integration Pvt. Ltd.

# Revision History

| Version | Date | Description | Author |
| --- | --- | --- | --- |
| 1.0 | 26 August 2026 | Initial release | [Author] |

- Table of Contents

Revision History	2

1 Introduction	4

1.1 Purpose	4

1.2 Scope	4

1.3 How to Read the Call-Hierarchy Trees	4

1.4 Definitions and Abbreviations	4

1.5 References	5

2 System Overview	5

3 Firmware Architecture and Call Hierarchy	5

3.1 File Layout	5

3.2 Startup Call Hierarchy	5

3.3 Runtime Threads	6

3.4 Function Reference: app/ (tasks.c, proto.c)	6

3.5 Function Reference: test/ and cards/	7

3.6 Runtime Call Hierarchy: RES RUN	8

3.7 Notable Implementation Details	9

4 GUI Architecture and Call Hierarchy	10

4.1 File Layout	10

4.2 Startup Call Hierarchy	10

4.3 Function/Method Reference	10

4.4 Runtime Call Hierarchy: Button Press to Wire	11

4.5 Runtime Call Hierarchy: Incoming Line to Screen Update	11

4.6 Notable Implementation Details	12

5 Firmware-GUI Interface (Serial Protocol)	12

5.1 Commands	12

5.2 Events and Error Codes	13

6 Key Design Decisions	13

7 Known Limitations	14

Appendix A: System Reference	15

Appendix B: Glossary	15

# Introduction

## Purpose

This document explains how the HT_MK1 firmware and GUI source code are structured: which files call which functions, in what order, and what each function does. It is written for an engineer who will read, maintain, or extend the code, using the delivered source alongside this document.

## Scope

Covers the embedded firmware (Core/Inc, Core/Src) and the GUI application (gui_flutter/lib). For hardware design, refer to the project schematics. For operating the system, refer to the User Manual. For programming a unit, refer to the Flashing Procedure. For acceptance testing a built unit, refer to the Functional Test Procedure.

## How to Read the Call-Hierarchy Trees

Sections 3 and 4 include call trees in this form:

caller()
├── firstCallee()
│   └── nestedCallee()
└── secondCallee()

Each line is a function call made by the function above it, indented one level per call depth.

Every function name in a tree or table is exactly as it appears in the delivered source and can be searched for directly.

## Definitions and Abbreviations

| Term | Meaning |
| --- | --- |
| MCU | Microcontroller Unit: the single processor chip that runs the firmware. |
| RTOS | Real-Time Operating System. This project uses FreeRTOS: several software threads run on the one MCU. |
| I2C, SPI | Digital communication buses used between chips on the boards. |
| ADC / DAC | Analog-to-Digital / Digital-to-Analog Converter. |
| IDAC | Current-source output of the resistance-measurement ADC; pushes a known test current through a wire. |
| PGA | Programmable Gain Amplifier: sets how much a small signal is amplified before it is measured. |
| Netlist | The list of expected wire connections for the harness under test. |
| Fixture | Which physical connector the harness is currently on: the Matrix connector or the HV connector. |

## References

Project schematics: Control Card, Matrix Card, HV Card.

HT_MK1 User Manual, Flashing Procedure, Functional Test Procedure.

# System Overview

The HT_MK1 tests up to 256 wires in a harness in three stages: continuity, 4-wire (Kelvin) resistance, and 500 V insulation. The software is two programs that talk over a serial link:

| Component | Runs on | Role |
| --- | --- | --- |
| Firmware | One STM32G474 MCU, on the Control Card | Owns all hardware. Runs every test. The Matrix Card and up to four HV Cards have no processor of their own; the firmware drives them over I2C/SPI. |
| GUI | Windows desktop (Flutter) | Sends commands, receives results, manages netlists. No test logic of its own; it is a client of the firmware's serial protocol (Section 5). |

# Firmware Architecture and Call Hierarchy

## File Layout

| Folder | Files | Layer |
| --- | --- | --- |
| drivers/ | mcp23017, ads124s08, ad7476, dac8830, ds18b20, ads1232 (bench-only, compiled out by default) | Chip-level. Talks to hardware registers over I2C/SPI/GPIO. No test logic. |
| cards/ | matrix_card, hv_card, control_frontend | Board-level. Combines drivers into card operations (route a pin, close a relay, set a mode). |
| test/ | continuity, kelvin, insulation | Test-level. One algorithm per file, built from card-level calls. |
| bsp/board | board | Start-up. Instantiates and initializes every driver/card instance used by the layers above. |
| app/ | tasks, proto, log | Application-level. RTOS threads, the command queue, the serial protocol, logging. |

Each layer only calls into the layer(s) below it: app/ calls test/ and cards/, test/ calls cards/ and drivers/, cards/ calls drivers/. Nothing in drivers/ or cards/ calls upward into test/ or app/.

## Startup Call Hierarchy

main()                                              Core/Src/main.c
├── HAL_Init(), SystemClock_Config()
├── MX_I2C2_Init(), MX_I2C3_Init(), MX_SPI1_Init(), MX_SPI2_Init(), MX_SPI3_Init(), ...
│                                                    (CubeMX-generated peripheral setup)
├── Board_Init()                                    bsp/board.c
│   ├── board_init_matrix()
│   ├── board_init_ads124s08()
│   ├── board_init_frontend()
│   ├── board_init_hv(i)                            once per fitted HV card (BOARD_HV_COUNT)
│   └── board_init_temp()                           failure here is non-fatal
├── osKernelInitialize()
├── MX_FREERTOS_Init()
│   └── Tasks_Init()                                app/tasks.c
│       ├── Log_HwInit_LPUART1(), Log_Init()
│       ├── osMutexNew()          -> s_hwmtx        (the one hardware-access mutex)
│       ├── osMessageQueueNew()   -> s_cmdq, s_rxq
│       └── osThreadNew() x4      -> tLogger, tSafety, tSequencer, tComms
└── osKernelStart()                                 (the four threads above begin running)

Core/Src/main.c, Core/Src/app_freertos.c, Core/Src/bsp/board.c, Core/Src/app/tasks.c.

Board_Init() aborts the whole chain on the first failure, except board_init_temp(): a missing temperature sensor must not stop the electrical-test hardware from coming up. Board_IsReady() records the outcome; proto.c checks it before any hardware command.

## Runtime Threads

| Thread | Priority | Entry function | Role |
| --- | --- | --- | --- |
| tSafety | High | SafetyTask() | Always-on loop. Forces every card to a safe state on a latched fault, and backstops HV-off whenever no test is running. |
| tComms | AboveNormal | CommsTask() | Reads bytes from the UART queue and feeds the protocol parser. Kept above tSequencer so a command, especially ABORT, is always received even mid-test. |
| tSequencer | Normal | SequencerTask() | Blocks on the command queue; runs one command at a time, holding the hardware mutex for its duration. |
| tLogger | Low | Log_Task() | Drains the debug log queue to the UART. |

## Function Reference: app/ (tasks.c, proto.c)

| Function | Calls | Purpose |
| --- | --- | --- |
| Tasks_Init() | Log_HwInit_LPUART1(), osMutexNew(), osMessageQueueNew() x2, osThreadNew() x4 | One-time boot setup: logger, hardware mutex, command/rx queues, the four RTOS threads. |
| SafetyTask() | force_safe_all(), HvCard_HvEnable() | Runs forever. See the tree in Section 3.2 caption and the snippet below. |
| SequencerTask() | run_command() | Runs forever. Blocks on s_cmdq; hands each dequeued command to run_command(). |
| run_command() | Continuity_TestPair(), Kelvin_MeasurePair(), Insulation_TestPair(), run_continuity_all(), run_resistance_all(), run_insulation_all(), Board_ScanBus(), DS18B20_ReadTemperature(), force_safe_all() | Dispatches one dequeued command by type (switch statement); acquires s_hwmtx first. |
| run_resistance_all() | Kelvin_MeasurePair(), Proto_EvtRes(), Proto_EvtProgress(), Proto_EvtDone() | Loops the loaded netlist; one Kelvin measurement per pair; streams results as they complete. |
| run_continuity_all() | Continuity_TestPair(), Proto_EvtCont(), Proto_EvtProgress(), Proto_EvtDone() | Same shape as run_resistance_all(), for continuity (verify mode, or full discovery scan). |
| run_insulation_all() | Insulation_TestPair(), Proto_EvtInsul(), Proto_EvtProgress(), Proto_EvtDone() | Same shape again, for insulation. Requires INSUL ARM to have been accepted first. |
| CommsTask() | Proto_RxByte() | Runs forever. Blocks on the byte queue fed by the UART receive interrupt. |
| Proto_RxByte() | proto_exec() | Assembles bytes into a line; calls proto_exec() once a full line (ending '\n') is buffered. |
| proto_exec() | proto_split(), proto_hw_ready(), proto_post(), proto_post_run() | Parses one command line and dispatches it by string comparison. See Section 3.6. |
| proto_post_run() | proto_post() | Refuses with ERR EBUSY if a run is already active; otherwise clears the abort flag and posts. |
| proto_post() | Tasks_PostCommand() | Queues one command; replies <OK started, or ERR EBUSY if the queue is full. |

## Function Reference: test/ and cards/

| Function | Calls | Purpose |
| --- | --- | --- |
| Continuity_TestPair() | MatrixCard_ConnectPair(), Frontend_SetMode(), AD7476_ReadVolts() | Routes one wire pair, reads a voltage, judges open/connected/anomaly by voltage band. |
| Kelvin_MeasurePair() | MatrixCard_SetSensePaired(), MatrixCard_ConnectPair(), Frontend_SetMode(), ADS124S08_SetIdac(), ADS124S08_SetMux(), ADS124S08_SetGain(), ADS124S08_BypassPga(), ADS124S08_ConvertOnce(), ADS124S08_OhmsRatiometric(), ADS124S08_OhmsFromCurrent() | Full 4-wire, auto-ranged, ratiometric resistance measurement. See Section 3.7 for the exact call order. |
| Insulation_TestPair() | HvCard_ConnectPair(), HvCard_SetVoltageFraction(), AD7476_ReadVolts() (via HvCard_ReadLeakageVolts()), HvCard_HvOff() | Cold-switches relays, applies HV, reads the leakage node, always de-energizes and opens relays on exit. |
| MatrixCard_ConnectPair() | MCP23017_WritePin() | Closes the mux channels routing one HI and one LO pin to the shared force/sense buses. |
| HvCard_ConnectPair() | MCP23017_WritePin() | Closes one inject relay and one return relay on the addressed HV card. |
| Frontend_SetMode() | HAL_GPIO_WritePin() (OPT0_CNTR) | Switches the shared analog front end between continuity-divider and impedance mode. |

## Runtime Call Hierarchy: RES RUN

Command arrival (tComms) and command execution (tSequencer) are two separate call chains, joined by the command queue:

CommsTask()                                         app/tasks.c
└── Proto_RxByte()            [once per received byte]     app/proto.c
    └── proto_exec("RES RUN")                 [called on the terminating '\n']
        ├── proto_hw_ready()  -> Board_IsReady()
        └── proto_post_run(CMD_RES_RUN)
            └── proto_post()
                └── Tasks_PostCommand()  -> osMessageQueuePut(s_cmdq)
                                             reply sent: <OK started
 
SequencerTask()                                     app/tasks.c
└── run_command()             [dequeues the command; holds s_hwmtx for the whole run]
    └── run_resistance_all()
        └── Kelvin_MeasurePair()      [once per netlist pair]      test/kelvin.c
            ├── MatrixCard_SetSensePaired() / MatrixCard_ConnectPair()   cards/matrix_card.c
            ├── Frontend_SetMode(FRONTEND_MODE_IMPEDANCE)                cards/control_frontend.c
            ├── ADS124S08_SetIdac(2 mA) -> settle -> ConvertOnce()       (excited reading, DUT)
            ├── ADS124S08_SetMux(AIN8) / BypassPga() / ConvertOnce()     (excited reading, reference R131)
            ├── ADS124S08_SetIdac(OFF) -> settle -> ConvertOnce() x2     (zero-current baseline, both channels)
            └── ADS124S08_OhmsRatiometric()   or   ADS124S08_OhmsFromCurrent()  (fallback)
        Proto_EvtRes() + Proto_EvtProgress()         [sent after every pair]
        ... repeats for every pair in the netlist, checking Proto_AbortRequested() each time ...
        Proto_EvtState("idle") -> Proto_EvtDone("res", pass, fail)

Verified against Core/Src/app/tasks.c (run_command(), run_resistance_all()), Core/Src/app/proto.c (proto_exec(), proto_post_run(), proto_post()), and Core/Src/test/kelvin.c (Kelvin_MeasurePair()).

CONT RUN and INSUL RUN follow the identical two-chain shape: proto_exec() posts to the queue, and SequencerTask() dequeues and calls run_continuity_all() or run_insulation_all(), substituting Continuity_TestPair() or Insulation_TestPair() for Kelvin_MeasurePair() inside the loop. INSUL RUN additionally requires INSUL ARM to have set the armed flag first, checked before the loop starts.

ABORT does not go through the command queue at all. proto_exec() sets a flag inline and replies immediately, because tSequencer holds s_hwmtx for the whole run and a queued command could not preempt it. The run loops (run_resistance_all(), etc.) poll this flag after every pair.

## Notable Implementation Details

SafetyTask() is the entire independent safety backstop, unconditional on what the sequencer is doing:

static void SafetyTask(void *arg)
{
  for (;;)
  {
    if (s_fault)
    {
      if (osMutexAcquire(s_hwmtx, 50U) == osOK)
      {
        force_safe_all();
        osMutexRelease(s_hwmtx);
      }
    }
    else if (!s_hv_active)
    {
      for (i = 0; i < BOARD_HV_COUNT; i++)
        HvCard_HvEnable(&g_hv[i], 0U);   /* backstop: HV off whenever idle */
    }
    osDelay(10U);
  }
}

tasks.c, SafetyTask() (from line 152).

Kelvin_MeasurePair() implements the ratiometric calculation this test exists to make possible:

if (ref_ok && (ref_excited - ref_zero) != 0)
{
  res->resistance_ohm = ADS124S08_OhmsRatiometric(
      res->code, gain_dut, ref_excited - ref_zero, ADS124S08_GAIN_1,
      KELVIN_CAL_R_REF_OHM);
  res->ratiometric = 1U;
}
else
{
  res->resistance_ohm = ADS124S08_OhmsFromCurrent(&g_ads124s08, res->code,
                                                   KELVIN_FORCE_CURRENT_A);
}

kelvin.c, Kelvin_MeasurePair() (from line 190).

# GUI Architecture and Call Hierarchy

## File Layout

| Folder | Files | Layer |
| --- | --- | --- |
| htproto/ | codec.dart, messages.dart, connection.dart, serial_transport.dart, simulator.dart, netlist_file.dart | Wire protocol only. Encodes commands, parses replies/events, manages the connection. No UI code. |
| design/ | tokens.dart, widgets.dart, painters.dart, model.dart | Presentation primitives: colors, reusable widgets, canvas painters (wiring diagram, histogram). |
| app/ | app_state.dart, shell.dart, parts.dart, modals.dart, report.dart, run_history.dart, build_tag.dart | Application state and cross-screen chrome (status bar, nav rail). |
| views/ | run_view.dart, cont_view.dart, res_hv_views.dart, misc_views.dart | The seven screens. Each is a thin layer that reads AppState and calls its methods; no protocol code here. |

## Startup Call Hierarchy

main()                                              lib/main.dart
├── parse launch options (--serial / --sim / --host / --port / --scenario / --nets)
├── AppState(...)             [constructed once; holds every screen's state]
├── state.refreshPorts()
└── runApp(HtApp(state: state, ...))
    └── HtHome  (StatefulWidget)                     lib/main.dart
        └── AnimatedBuilder(animation: state, ...)   [rebuilds on every state.notifyListeners()]
            └── AnimatedSwitcher(child: <the view named by state.view>)

lib/main.dart. There is no router; the visible screen is just a string field on AppState.

## Function/Method Reference

| File : Function/Method | Calls | Purpose |
| --- | --- | --- |
| app_state.dart : runCont() / runRes() / runHv() | _startRun(), _cmd() | Per-test entry points, called directly from each view's Run button. |
| app_state.dart : _startRun() | _cmd(), _beginRun() | Sends the command; only marks the run "started" (running=kind) after the instrument's reply is accepted. |
| app_state.dart : _cmd() | ConnectionManager.execute() | Thin wrapper: calls execute() and reports a refusal if the reply is an error. |
| app_state.dart : onEvent() | _onCont(), _onRes(), _onInsul(), _onProgress(), _onDone(), notifyListeners() | Dispatches every incoming ! event by its runtime type. See Section 4.5. |
| connection.dart : execute() | _execute() | Public entry point; returns the first (and normally only) reply message. |
| connection.dart : _execute() | _sendLine() | Registers a pending-reply entry, sends the command, awaits it with a timeout Timer. |
| connection.dart : _sendLine() | Transport.send() | Writes the command bytes to the transport; logs and mirrors them to the wire log. |
| connection.dart : _handleIncoming() | parseLine(), onEvent callback | Called once per complete received line; routes it as a reply match or an event. |
| codec.dart : parseLine() | _parseReply(), _parseEvent() | Dispatches on the leading byte ('<' or '!') and builds a typed Message subclass. |
| netlist_file.dart : _guessFixture() | _guessShape() | Infers connector layout (DB9/DB37/circular/rect/etc.) from a netlist's Conn ID / Part Number columns. |

## Runtime Call Hierarchy: Button Press to Wire

ContView "Run continuity" button                    views/cont_view.dart
└── AppState.runCont()                                app/app_state.dart
    └── _startRun('cont', commands.contRun(mode), ...)
        └── _cmd(wire)
            └── ConnectionManager.execute(wire)        htproto/connection.dart
                └── _execute(wire)
                    ├── _sendLine(wire) -> Transport.send()   [writes bytes, logs tx]
                    └── await entry.completer.future           [resolves on the matching '<' reply, 2 s timeout]
    (only once the reply resolves)
    -> _beginRun() sets running = 'cont' -> notifyListeners() -> screen redraws

Verified against gui_flutter/lib/app/app_state.dart and lib/htproto/connection.dart.

## Runtime Call Hierarchy: Incoming Line to Screen Update

Transport.incoming  (Stream<Uint8List>)              htproto/serial_transport.dart or simulator.dart
└── LineFramer.feed()                                  htproto/connection.dart
    └── _handleIncoming(line)         [once per complete '\n'-terminated line]
        ├── parseLine(line)                            htproto/codec.dart
        │   └── returns a typed Message  ('<' -> reply, '!' -> event)
        ├── if reply: matched to the oldest pending command -> completes its Future
        └── if event: AppState.onEvent(m)               app/app_state.dart
            └── switch (m)
                ├── ContResult()  -> _onCont(m)
                ├── ResResult()   -> _onRes(m)
                ├── InsulResult() -> _onInsul(m)
                ├── SafeEvent()   -> handling = 'safe'   (the ONLY place this is set true)
                └── ... one case per event type ...
                    -> notifyListeners() -> AnimatedBuilder rebuilds the visible screen

Verified against gui_flutter/lib/htproto/connection.dart (line 443) and lib/app/app_state.dart (line 1905).

## Notable Implementation Details

codec.dart : commands.contRun(), showing how a typed call becomes an exact wire command:

static Uint8List contRun(ContMode mode) =>
    _line('CONT RUN ${mode.wire}');   // writes:  >CONT RUN verify\n

codec.dart, commands.contRun() (line 268).

Netlist connector-ID handling (netlist_file.dart): a netlist file can reuse the same connector ID for two different roles: its own mated pair, and a reference from a different connector's row. Treating both as one combined pin range let a foreign reference silently widen a connector's guessed size. The fix tracks native and foreign observations separately:

class _ConnObs {
  int? nativeMin, nativeMax;         // this connector's own mated-pair rows
  int? foreignSrcMin, foreignSrcMax; // OTHER connectors' rows that merely
  int? foreignDstMin, foreignDstMax; // reference this connector's pins
}
// A connector's guessed range is built from nativeMin/nativeMax whenever
// a native block exists at all - foreign observations never widen it.

netlist_file.dart, _ConnObs (line 347) and the range logic that follows it.

# Firmware-GUI Interface (Serial Protocol)

Plain text lines over a serial connection. Every line starts with a marker:

| Marker | Meaning |
| --- | --- |
| > | A command sent from the GUI to the firmware. |
| < | A reply from the firmware, immediately after a command. Exactly one reply per command. |
| ! | An event sent by the firmware on its own: a result, a progress update, or a state change. |
| # | A human-readable log line, for display only. |

## Commands

| Command | Firmware handler | What it does |
| --- | --- | --- |
| PING / ID / STATUS | proto_exec() | Basic health check and version/state query. |
| SAFE | proto_exec() -> CMD_FORCE_SAFE | Forces HV off, all relays open. |
| ABORT | proto_exec() (inline, not queued) | Stops the current run immediately. |
| NETLIST BEGIN/ADD/END/GET | proto_exec() | Loads the expected-wiring list, up to 256 pairs. |
| CONT RUN | run_continuity_all() | Continuity over the netlist, or full discovery scan. |
| RES RUN | run_resistance_all() | Resistance over the netlist. See Section 3.6. |
| INSUL ARM / INSUL RUN | run_insulation_all() | Arm, then run, the insulation test. |
| LIMITS GET / SET | proto_exec() | Reads/sets the resistance and insulation pass/fail limits. |
| FAULT CLEAR | proto_exec() -> CMD_CLEAR_FAULT | Clears a latched fault. |
| MANUAL PATH / MANUAL SWEEP | run_manual_sweep(), CMD_CONTINUITY | Engineering: test one pair, or one pin against all others. |
| MANUAL RELAY | proto_exec() (always refused) | Always ERR EHW: driving one HV relay by hand is not permitted. |
| FIXTURE | Proto_SetFixture() | Tells the firmware which connector the harness is on. |

## Events and Error Codes

| Event | Meaning |
| --- | --- |
| !PROGRESS | How far through the current run. |
| !CONT / !RES / !INSUL | One result, sent as soon as it is available. |
| !DONE | The run finished; pass/fail summary. |
| !STATE | Overall state; also a periodic heartbeat when idle. |
| !SAFE | The only signal the GUI treats as "safe to handle". |
| !FAULT | A fault occurred, with a code and text. |

| Code | Meaning |
| --- | --- |
| EBUSY | A run is already in progress, or the queue is full. |
| EFIXTURE | Wrong connector for this command. |
| ENOTARMED | Insulation requested without arming first. |
| ERANGE | Argument out of range. |
| EHW | Hardware not ready. |
| ESYNTAX | Command not understood. |

# Key Design Decisions

4-wire (Kelvin) resistance measurement

An earlier design measured resistance using two connections only, so switch resistance added error to every reading. A second, separate sense path was added that carries no current, so it adds no error. See MatrixCard_SetSensePaired() in Section 3.5.

Measuring resistance as a ratio against a reference resistor

The test current's own accuracy is not high enough on its own. Kelvin_MeasurePair() also reads a precision reference resistor using the same current and computes the result as a ratio (ADS124S08_OhmsRatiometric(), Section 3.7); this cancels the current source's error out of the result entirely.

Insulation test method

Insulation_TestPair() applies HV to one wire while grounding the rest and reads the leakage. Relays are only switched while HV is off (HvCard_ConnectPair() is always called before HvCard_SetVoltageFraction(), never after), to protect the relay contacts.

Shared bus addressing across cards

The Matrix Card and every HV card use identical chip addresses on their shared I2C bus. Exactly one card's connection to that bus is enabled at a time. See board_init_hv() in Section 3.2.

Abort must work immediately, even mid-run

tComms is given a higher priority than tSequencer specifically so ABORT is always received and acted on during a long run. See Section 3.3 and the ABORT note in Section 3.6.

# Known Limitations

Implemented in the current code but not yet verified against real hardware:

The ratiometric resistance measurement (ADS124S08_OhmsRatiometric() path) has not been checked against a wire of known resistance on real hardware.

The insulation (HV) test has not been run against real hardware through the GUI.

Board_ScanBus() has not been verified on real hardware.

DS18B20_ReadTemperature() has not been tested against a physical sensor.

board_init_hv() currently supports up to three HV cards (BOARD_HV_COUNT <= 3); a fourth requires a hardware connection that has been agreed but not yet built.

# Appendix A: System Reference

Peripheral and pin assignments, for an engineer adapting the firmware to a hardware revision.

| Peripheral | Used for | Notes |
| --- | --- | --- |
| I2C bus 1 | Matrix Card switching control | Local to the Control Card only, not shared. |
| I2C bus 2 | HV card relay control; Matrix Card resistance-ADC control | Shared. Exactly one card enabled at a time. |
| SPI bus 1 | Resistance-measurement ADC (Matrix Card) | 24-bit precision ADC (ADS124S08). |
| SPI bus 2 | Per-HV-card: HV output DAC and two sensing ADCs | Isolated for safety. |
| SPI bus 3 | Continuity/resistance front-end ADC (Control Card) | 12-bit ADC (AD7476). |
| 1-Wire (bit-banged GPIO) | Board temperature sensor | Diagnostic only; does not gate readiness. |

| Driver | Chip type | Role |
| --- | --- | --- |
| mcp23017 | 16-bit I/O expander (I2C) | Drives relay coils and switching-chip control lines. |
| ads124s08 | 24-bit precision ADC (SPI) | Resistance measurement; also supplies the test current. |
| ad7476 | 12-bit ADC (SPI) | Continuity sensing; HV rail and leakage sensing. |
| dac8830 | 16-bit DAC (SPI) | Sets the high-voltage output level. |
| ds18b20 | Digital temperature sensor (1-Wire) | Board temperature monitoring. |

# Appendix B: Glossary

| Term | Meaning |
| --- | --- |
| Stage 1 / Stage 2 | Continuity and resistance testing, harness on the Matrix connector. |
| Stage 3 | Insulation testing, harness on the HV connector. |
| Force connection | A connection that carries the test current. |
| Sense connection | A connection that measures voltage only, carrying no current. |
| Arm | Explicit confirmation required before high voltage can be applied. |